	[image: image1.png]CESKA
o o:chooni
INSPEKCE




	PRESS RELEASE


CTIA banned dangerous Christmas LED chain
(Prague, 15 May 2023) The Czech Trade Inspection Authority has found a product in the domestic market that may pose a serious risk of electric shock to consumers. The product in question is offered as LED Light Chain, the type/model number is AG-250LED.

The Czech Trade Inspection Authority has found that there is electrical equipment in the market in the Czech Republic that may pose a risk to consumers. The product is a LED light chain with non-replacable light sources, equipped with an electronic unit for the creation of light effects and a two-pole fork in a paper cardboard packaging. A paper label is placed on the product, stating CE, 220-250V, 50Hz, 11W, Bulb type 3V, 0.03W, 0.01A, Number of bulbs: 250pcs, AG-250LED, mark for TOII, IP20. On the packaging, it states 250LED Christmas light chain AG-250LED, 25m+1.5m with 8 programs, made in China, Distributor Lamer Fashion s.r.o., Libušská 319/126, Prague 4, 142 00, Czech Republic, CE and a pictogram not suitable for children under 3 years. 

The Czech Trade Inspection Authority found a serious risk in the product, as the power supply lead is not securely anchored to the electronic lighting effects unit and not sufficiently secured against pulling out (connected only by fusible material and soldering). If the lead is tampered with, the lead may be pulled out of the unit, thereby exposing live parts of the product. If the user touches this live part, which is energized, and a serious injury or death to the user by electric shock will result. A further serious risk of injury may be posed by the mechanical strength of the electronic lighting effects unit being insufficient. The cover can easily be removed from the control unit by hand without force or tools, thereby exposing live parts of the product. Also in this case, if the user touches a live part of the product, a serious injury or death to the user by electric shock will result.

The Czech Trade Inspection Authority has imposed a ban on the seller from placing the product on the market or it has already been placed on the market. The product has been reported to the MIT contact point and information about it has been published in the Safety Gate system.  

Product information:

Product name:  
                                         

Světelný řetěz LED diody

Model type/ number:



světelný řetěz / AG-250LED

EAN:





not displayed
Identification of the manufacturer:                        not displayed
Identification of the importer: 

              not displayed
Distributor:
           



VIETPRO s.r.o., Sokolovská 2120/78, Plzeň

Country of origin:



China

[image: image2.jpg]


           [image: image3.jpg]


               
Contact:       Spokesperson of the CTIA
                                                            Phone:
+420 296 366 233  
                       Mgr. Jiří Fröhlich
                                                            Mobile: 
+420 602 105 376

                                                            E-mail:
mluvci@coi.cz

