|  | PRESS RELEASE |
| --- | --- |

CTIA banned a dangerous light chain in the market

(Prague, 15 June 2023) The Czech Trade Inspection Authority has found that there is electrical equipment in the domestic market that may pose a risk to consumers. It is a light chain offered as LED MIX colour bulbs.

The Czech Trade Inspection Authority has found that there is an electrical device in the Czech market which may pose a risk to consumers. It is a light chain (Christmas lighting) intended for indoor use only with non-replaceable LED light sources equipped with an electronic unit for lighting effects in a paper cardboard packaging. CE, 250V, English instruction manual for the electronic unit switch (8 functions). On the packaging it says: LED, ITEM: 1101060 240L, EAN 8000011010762, CE, MUX color bulbs/green wire, 25000 hour average bulb life, instructions for use are in 5 foreign languages, capacity 55W, frequency 60Hz, FATO IN CINA, 220V, 110V, iTEM NO: 1101076.

The Czech Trade Inspection Authority has identified a serious risk of electric shock or death to the user if the user touches accessible live parts of the product as a result of the power supply lead being removed from the electronic unit or the electronic unit cover being removed. The power lead is not securely anchored to the electronic unit and is not sufficiently secured against pulling out (only fusible link and soldering). Manipulation of the lead may result in the lead being pulled out of the unit, thereby exposing live parts of the product. Furthermore, the mechanical strength of the electronic unit is not sufficient, the cover can be easily removed from the driver by hand without force or tools, which will also result in access to live parts of the product.

The Czech Trade Inspection Authority has imposed a protective measure on the seller (Article 50(3) of Act No. 90/2016 Coll.), which is a ban on the supply to the market of the said product, which is dangerous or has already been placed on the market. The product was reported to the contact point at the Ministry of Industry and Trade and information about it was published in the Safety Gate system.

Product information:

Product name: 			LED MIX color bulbs

Model type/ number:		LED Světelný řetěz/neuvedeno

EAN:				8000011010762

Identification of manufacturer:	neuvedeny

Identification of importer:	neuvedeny

Distributor: 			Tuan Đat Bui, Mlýnská 548, 36452 Žlutice

Country of origin: 			Čína