|  | PRESS RELEASE |
| --- | --- |

Dangerous Christmas tree-shaped chain

(Prague, 22 September 2023) The Czech Trade Inspection Authority has found a Christmas tree-shaped light chain on the domestic market that can cause serious electric shock to the user. This chain is sold as Strom svítící, the type/model number is LED Light Chain/T17791K144.

The Czech Trade Inspection Authority has identified electrical equipment in the market in the Czech Republic that may pose a risk to consumers. It is a Christmas tree-shaped light chain with non-replaceable LED light sources equipped with an electronic unit in a transparent plastic packaging. The product is CE marked, 2.5A, 250V, AC mark, LX-01, 12V-240V, 50/60Hz. A label is attached to the power cord stating CE, mark for protection class II, info in German. A label is glued on the packaging, where it says: made in P.R.C, ITEM NO.: T17791K144, LIGHTING TREE, Distributor.

The product has a serious safety risk for the consumer as the power supply lead is not securely anchored to the electronic unit and sufficiently secured against pulling out (connected only by fusible material and soldering). If the lead is tampered with, the lead may be pulled out of the unit, exposing live parts of the product. In addition, the mechanical strength of the electronic unit is not sufficient as the cover can be easily removed from the driver by hand without using any force or tool, thereby exposing live parts of the product. If the user touches live parts of the product that are live, serious injury or death to the user by electric shock could occur.

The Czech Trade Inspection Authority imposed a protective measure on the seller (pursuant to Section 50(3) of Act No. 90/2016 Coll., on Conformity Assessment of Selected Products when Made Available in the Market), which is a ban on the supply of this product to the market, including the product that has already been supplied to the market. The product has been reported to the contact point at the MIT and information about it has been published in the Safety Gate system.

Product information:

Product name:  	                                        Strom svítící

Product type / number:				LED Světelný řetěz/T17791K144

EAN:						8597791180001

Identification of manufacturer:            	not displayed

Identification of importer:                  		not displayed

Distributor:	              Vân Sáu Lé, Majakovského 230, 36235 Horní Blatná

Country of origin:	              		China